# Cardiac MRI Reference Converter — Calculations

Live tool (open in a browser): https://nethahussain.github.io/cardiac-mri-reference-converter/

Documentation of every calculation performed by the tool. Reference ranges are the 95% prediction intervals from Raisi-Estabragh et al. (2024), Healthy Hearts Consortium (JACC Cardiovasc Imaging; doi:10.1016/j.jcmg.2024.01.009).

# 1. Body surface area (BSA) — Mosteller formula

BSA = √(weight × height / 3600), with weight in kg, height in cm, and the result in m².

Example: 73 kg, 180 cm → √(73 × 180 / 3600) = √3.65 = 1.91 m².

# 2. Age and age group

Age is the difference between the current date and the date of birth (reduced by 1 if this year's birthday has not yet occurred). Age is then mapped to a reference age band:

| Age | Age group |
| --- | --- |
| 18–29 | 18-29 |
| 30–39 | 30-39 |
| 40–49 | 40-49 |
| 50–59 | 50-59 |
| 60–69 | 60-69 |
| 70+ | 70+ |

Below 18 years: no reference group available. Some ethnicity groups also lack data for the younger bands; in those cases values are shown without a reference comparison.

# 3. Stroke volume (SV)

SV = EDV − ESV, computed separately for the left (LV) and right (RV) ventricle. EDV = end-diastolic volume, ESV = end-systolic volume.

# 4. Ejection fraction (EF)

EF = (SV / EDV) × 100 = ((EDV − ESV) / EDV) × 100, in percent — the fraction of blood pumped out per beat.

# 5. Cardiac output (CO)

CO = SV × heart rate / 1000, in l/min (SV in ml, heart rate in beats/min). Cardiac output is only shown when a heart rate is provided.

# 6. Body-size indexing

Each volume and mass is normalised to body size. The tool offers two indexing methods, matching the two body-size corrections in the publication:

| Method | Formula | Units |
| --- | --- | --- |
| Body surface area | value ÷ BSA | ml/m², g/m² |
| Height | value ÷ height (m) | ml/m, g/m |

# 7. Atrial ejection fraction

LAEF and RAEF = (V_max − V_min) / V_max × 100, using the maximal and minimal atrial volumes, in percent.

# 8. Segmentation methods

The publication provides reference ranges for three predefined segmentation protocols. They differ chiefly in what is included in the myocardial mass (and, to a lesser extent, in the cavity volumes). Select the method that matches how your measurements were obtained.

| Method | Definition |
| --- | --- |
| Smooth | Smooth endocardial contours; papillary muscles and trabeculae excluded from the LV mass. |
| Papillary | Smooth endocardial contours with papillary muscles (but not trabeculae) included in the LV mass. |
| Anatomical | Both papillary muscles and trabeculae included in the LV mass. |

Mass therefore increases from smooth → papillary → anatomical, while the corresponding cavity volumes decrease slightly.

# 9. Reference comparison

Each computed value is compared with the [min, max] range for the patient's ethnicity, sex, age group, segmentation method and indexing:

| Condition | Status | Flag |
| --- | --- | --- |
| value < min | Low | * (asterisk) |
| min ≤ value ≤ max | Normal | none |
| value > max | High | * (asterisk) |

# 10. Visual gauge

The gauge maps a value to a percentage position: padding = (max − min) × 0.35; gauge_min = min − padding; gauge_max = max + padding; position = ((value − gauge_min) / (gauge_max − gauge_min)) × 100%, clamped to 0–100%. The green band marks the normal range.

# Workflow summary

Patient data is entered (sex, ethnicity, date of birth, weight, height, heart rate).

BSA is calculated with the Mosteller formula.

Segmentation method and indexing are selected.

Measurements are entered for each chamber (atria optional).

SV, EF and CO are derived; atrial EF is derived from max and min volumes.

All volumes and masses are indexed by BSA or height.

Reference ranges are looked up by ethnicity, sex, age group, segmentation and indexing.

Out-of-range values are flagged, and a report is generated for copying.